# Tag Now Company

# Guidelines - Do's and Don'ts

Do's

- Place the RF EAS tag on a flat surface

- Drop into the packaging.

Don'ts

- Touch the adhesive side of the tag

- Bend the tag over 90o

- Place an RF EAS tag on top of another RF EAS tag.

- Use marked labels. These tags are designated as faulty. All Tag Now labels go through a quality test prior to leaving the factory. Any tag within the roll that is found to be "marginal or defective during this process is identified with a black dash and compensated for by the addition of labels into a roll. There will always be 2,000 operable labels in a roll.

- Tag directly onto metals. This will shield the tag from working. Note: Create a 7mm space from the metal if tagging on a product consisting of metal. e.g.; battery compartment can be a safe area or if the product has plastic covering.

- Supply RF EAS source tagged merchandise to retailer without RF EAS systems.

Do not cover important product information with RF EAS labels

- Lot numbers

- Product details and warnings

- Ingredients

- APN Barcodes

Detailed Source Tagging Procedures:

- What type of packaging is used ?

e.g. typical packaging styles include:

- Blister Card Single layer,

- Blister Card double layer

- Clam Shell with insert panel – single layer

- Clam Shell with insert panel – double layer

- Plastic Container

- Identify the materials of construction of the packaging

Typical materials include:

- Printed paper

- Plastic films

- Foil laminated paper board

- Injection molded bottles

- Identify the final product and packaging that is placed on the retailer’s shelf

for purchase by the consumer

4.  Locate the barcode.

The RF security label needs to be within a 3” radius of the barcode, to ensure the deactivation of the RF security label. The orientation of the label with respect to the barcode is not critical.

- What is the product ?

The product itself may affect the functionality of the RF security label

- Develop source tagging options

- Locate the security label within the product itself

- Locate the security label on the product itself

- Source tag inside the product’s packaging

Tag Now provides recommendations according to each product !!